- IntelliGuide Plug-In for TCS

- Version 1.0, May 2024

# Overview

This document describes the IntelliGuide Plug-In for the Precise Automation TCP Command Server (TCS) application.

The stereo location capabilities of this plug-in requires the IntelliGuide enabled head on the robot.

To use this plug-in, load the Tcp_cmd_server_pa project, which consists of the standard TCS plus the plug-in files: StereoVision.gpl, Lib_Functions.gpp, Lib_stereocal.gpp,  and Lib_StereoLocator.gpp.

# IntelliGuide Commands

The table below describes the commands contained in this plug-in.

See the TCP Command Server User's Guide for information about the standard TCS commands.

| Command Case ignored | Arguments | Description, responses are for PC mode. Telnet mode is similar but more descriptive. |
| --- | --- | --- |
| StereoLocate | arg1: Robot Number (starting at 1). arg2: The camera number. 1 = Front Facing, 2 = Downward Facing | Perform a stereo location for the specified robot and camera. |
| StereoParam | arg1: The robot number (starting at 1) arg2: The camera number. 1 = Front Facing, 2 = Downward Facing arg3: (Optional) Set the Vision Process Name arg4: (Optional) Set the Vision Tool Name arg5: (Optional) Set the Optimum Distance To Target arg6: (Optional) Set the Optimum Window Scale Factor arg7: (Optional) Set the Wrist Axis Index arg8: (Optional) Set the Aruco 1 Number arg9: (Optional) Set the Aruco 2 Number arg10: (Optional) Set the Distance Between Arucos arg11: (Optional) Set the Maximum Aruco Distance Estimated Error arg12: (Optional) Set the wait time between pictures | Read or write the stereoscopic parameters. If only arg1 and arg2 are supplied, it will read the parameters. If more are specified, it is considered a write operation. When writing the parameters, all parameters must be specified. |
| VResultInfoString | arg1: (Optional) The vision tool to access arg2: (Optional) The result number to access if arg1 is specified. | Reads the vision result string. If no arguments are specified, the result is from the last vision process executed. If arguments are specified, the specific result is returned. Response is the result InfoString property of VisResult |
| VTooLProperty | arg1: The name of the vision tool in the project arg2: The name of the property to read or write arg3: (Optional) If specified, the value of the property to set. | Read or writes the property of a vision tool. If arg3 is not specified, it is a read request. If arg3 is specified, it is a write request. |

## Custom hook routines

The following routines are called by TCS to perform custom operations.

### Hook_InitCommand

### This routine is called when a new command object is created for a robot.

# Appendix - Error Codes

TCS uses standard GPL error codes in the command responses.  They are described in the Precise Documentation Library.  The table below shows custom error codes that are specific to this plug-in.

| Number | Error message text and description |
| --- | --- |
| -4050 | * Arm Too Close * The robot arm is too close to the target and should be moved back. |
| -4051 | * Mid Plate Angle Too Large * The calculated angle difference between the located targets is too large. This should not occur. |
| -4052 | * Too Far Above Arucos * he robot arm is too far above the target and should be moved down. |
| -4053 | * Too Far Below Arucos* The robot arm is too far below the target and should be moved up . |
| -4054 | * Arm Too Far * The robot arm is too far from the target and should be moved closer |
| -4055 | * Too Close To Center* The robot arm is too close to the center when searching for ArUcos. |
| -4063 | * Teach Arucos Not Found * The ArUco or ArUcos were not located. |
| -4064 | *Cannot Locate Target * The ArUco or ArUcos were located but not within acceptable tolerances. |

# Appendix - Acquisition Tool Properties

- Display Name: Camera Number

Reference Name: cameranumber

Description: The camera the tool executes against

Category: Identity

Read Only: True

Type: Integer

- Display Name: Name

Reference Name: name

Description: The name of the tool instance

Category: Identity

Read Only: False

Type: String

- Display Name: Acquire Extension

Reference Name: acquireextension

Description: Acquire Extension (Playback and Record mode only)

Category: Advanced Operation

Read Only: False

Type: Enum BMP, PNG, JPG

- Display Name: Acquire Max Save Images

Reference Name: acquiremaxsaveimages

Description: Acquire Max Images (-1 = infinite)

Category: Advanced Operation

Read Only: False

Type: Integer

- Display Name: Acquire Mode

Reference Name: acquiremode

Description: "Normal Acquire" - acquires a single image from the specified camera and places it into a frame buffer.

"Acquire and Save" - performs the same operation as "Normal Acquire" and then saves the image to a disk file.

"Clear Buffer" - Sets all of the pixels in a buffer to the intensity value of zero.

"Play from Disk" - restores an image from a disk file.  The disk file is specified by the AcquirePath and AcquirePrefix (see below).  If there are multiple numbered image files with the same prefix, each time the Vision Process is executed, the contents of the next sequential file will be copied to the image buffer.

"Save Only" - stores the contents of the current frame buffer to a disk file.  This enables a GPL program to execute a vision process that saves selected frame buffers for later access.

Category: Advanced Operation

Read Only: False

Type: Enum NORMAL_ACQUIRE, ACQUIRE_AND_SAVE, PLAY_FROM_DISK, SAVE_ONLY, CLEAR_BUFFER

- Display Name: Acquire Path

Reference Name: acquirepath

Description: String that defines the path to the disk file when AcquireMode is set to "Acquire and Save" or "Play from Disk'".  If blank (""), the default path will be used. If the path does not contain a ':' (i.e. C:\...), AcquirePath will be appended to the path where the project files are stored"

Category: Advanced Operation

Read Only: False

Type: String

- Display Name: Acquire Prefix

Reference Name: acquireprefix

Description: String that defines the disk file name (excluding the required .BMP extension) when AcquireMode is set to "Acquire and Save" or "Play from Disk". This string is combined with AcquirePath, a numerical index, and the ".BMP" file extension to generate the disk file name for saving or loading vision images.

Category: Advanced Operation

Read Only: False

Type: String

- Display Name: Active Layer

Reference Name: activelayer

Description: Specifies the type of data to be loaded into the image frame buffer for processing by subsequent vision tools. For grayscale cameras, only "Monochrome" image buffers are available and each pixel in the frame buffer has a value from 0 to 255 that defines its grayscale intensity. For color cameras, the frame buffer can be loaded with one of four types of data:  monochrome, red, green or blue.  The monochrome mode produces the same results as a grayscale camera. The red, green and blue modes load the intensity data for a single color.  For example, if "RedLayer" is selected, the image frame buffer will contain values that indicate the intensity (response) of only the red receptors of the camera. Tools that are applied to this image will only be operating on the red color layer of the camera.  The one exception is the "Pixel Color Window" Tool.  When this tool is executed, it automatically accesses all three of the RGB color layers of an image and is unaffected by the setting of the ActiveLayer property.

Category: Advanced Operation

Read Only: False

Type: Enum Monochrome, RedLayer, GreenLayer, BlueLayer

- Display Name: Jpeg Quality

Reference Name: jpegquality

Description: Image quality for JPEG (.jpg) file save, from 0 to 100

Category: Advanced Operation

Read Only: False

Type: Integer

- Display Name: Monochrome Display

Reference Name: monochromedisplay

Description: Does the image display as a monochrome image (true) or as a color image (false).

Category: Advanced Operation

Read Only: False

Type: Boolean

- Display Name: Backlight Compensation

Reference Name: backlightcompensation

Description: The backlight compensation level.

Category: Video Properties

Read Only: False

Type: Integer

- Display Name: Brightnes

Reference Name: brightnes

Description: The brightness level

Category: Video Properties

Read Only: False

Type: Integer

- Display Name: Contrast

Reference Name: contrast

Description: The contrast level

Category: Video Properties

Read Only: False

Type: Integer

- Display Name: Hue

Reference Name: hue

Description: The hue level

Category: Video Properties

Read Only: False

Type: Integer

- Display Name: Saturation

Reference Name: saturation

Description: The saturation level

Category: Video Properties

Read Only: False

Type: Integer

- Display Name: Sharpness

Reference Name: sharpness

Description: The sharpness level

Category: Video Properties

Read Only: False

Type: Integer

- Display Name: Gamma

Reference Name: gamma

Description: The gamma level

Category: Video Properties

Read Only: False

Type: Integer

- Display Name: White Balance

Reference Name: whitebalance

Description: The white balance level

Category: Video Properties

Read Only: False

Type: Integer

- Display Name: Gain

Reference Name: gain

Description: The gain level

Category: Video Properties

Read Only: False

Type: Integer

- Display Name: Exposure

Reference Name: exposure

Description: The exposure level

Category: Video Properties

Read Only: False

Type: Integer

# Appendix – Fiducial Tool Properties

- Display Name: Camera Number

Reference Name: cameranumber

Description: The camera the tool executes against

Category: Identity

Read Only: True

Type: Integer

- Display Name: Name

Reference Name: name

Description: The name of the tool instance

Category: Identity

Read Only: False

Type: String

- Display Name: Height

Reference Name: height

Description: The height of the region of interest

Category: Placement / Size

Read Only: False

Type: Single

- Display Name: Width

Reference Name: width

Description: The height of the region of interest

Category: Placement / Size

Read Only: False

Type: Single

- Display Name: X Position

Reference Name: xpos

Description: The x coordinate of the center of the region of interest

Category: Placement / Size

Read Only: False

Type: Single

- Display Name: Y Position

Reference Name: ypos

Description: The y coordinate of the center of the region of interest

Category: Placement / Size

Read Only: False

Type: Single

- Display Name: Adaptive Threshold

Reference Name: adaptivethreshold

Description: Constant for adaptive thresholding before finding contours.

Category: Operation

Read Only: False

Type: Integer

- Display Name: Corner Max Iterations

Reference Name: cornerrefinementmaxiterations

Description: Maximum number of iterations for stop criteria of the corner refinement process

Category: Operation

Read Only: False

Type: Integer

- Display Name: Corner Min Accuracy

Reference Name: cornerrefinementminaccuracy

Description: Minimum error for the stop criteria of the corner refinement process

Category: Operation

Read Only: False

Type: Double

- Display Name: Corner Refinement

Reference Name: cornerrefinement

Description: The corner refinement method

Category: Operation

Read Only: False

Type: Enum None, SubPixel, Contour

- Display Name: Corner Window Size

Reference Name: cornerrefinementwindowsize

Description: Window size for the corner refinement process (in pixels)

Category: Operation

Read Only: False

Type: Integer

- Display Name: Dictionary

Reference Name: dictionaryconfiguration

Description: The dictionary to use for locating the fiducials

Category: Operation

Read Only: False

Type: Enum Dict4X4_250, Dict5X5_250, Dict6X6_250, Dict7X7_250, Dict7X7_1000

- Display Name: Relative Tool

Reference Name: relativetoolname

Description: If a tool is to be positioned relative to another tool, this is the name of the referenced tool.  Tools can be relative to any vision tool that returns a vision coordinate position.  By default tools are not relative to another tool and this property is empty.

Category: Operation

Read Only: False

Type: Custom Value based on Context

- Display Name: Result Color

Reference Name: resultdisplaycolor

Description: The color to use when rendering the result markers

Category: Results Settings

Read Only: False

Type: Enum Gold, Blue, Violet, Red, Black, White

- Display Name: Result Scale

Reference Name: resultdisplayscale

Description: The scale to apply to the size of the result display

Category: Results Settings

Read Only: False

Type: Double

- Display Name: Show Results

Reference Name: showresults

Description: Alters how the results of a tool is graphically displayed.  Each vision tool has a default method for display, e.g. a line or a frame.   This property allows the graphical display to be changed.  For example, a Fixed Frame Tool is normally displayed as a reference frame.  However, if it is being used as a datum line or a point, its display can be changed to a line or a point for visual clarity.  This property does not affect the actual results of the tool.

Category: Results Settings

Read Only: False

Type: Enum NONE, POINT, LINE, FRAME, ARC

# Appendix – Barcode Reader Tool Properties

- Display Name: Camera Number

Reference Name: cameranumber

Description: The camera the tool executes against

Category: Identity

Read Only: True

Type: Integer

- Display Name: Name

Reference Name: name

Description: The name of the tool instance

Category: Identity

Read Only: False

Type: String

- Display Name: Height

Reference Name: height

Description: The height of the region of interest

Category: Placement / Size

Read Only: False

Type: Single

- Display Name: Width

Reference Name: width

Description: The height of the region of interest

Category: Placement / Size

Read Only: False

Type: Single

- Display Name: X Position

Reference Name: xpos

Description: The x coordinate of the center of the region of interest

Category: Placement / Size

Read Only: False

Type: Single

- Display Name: Y Position

Reference Name: ypos

Description: The y coordinate of the center of the region of interest

Category: Placement / Size

Read Only: False

Type: Single

- Display Name: Relative Tool

Reference Name: relativetoolname

Description: If a tool is to be positioned relative to another tool, this is the name of the referenced tool.  Tools can be relative to any vision tool that returns a vision coordinate position.  By default tools are not relative to another tool and this property is empty.

Category: Operation

Read Only: False

Type: Custom Value based on Context

- Display Name: Codabar

Reference Name: codabar

Description: Codabar is a linear, symbology barcode that represents data using a series of bars and spaces of varying widths. It was developed in 1972 and is commonly used in libraries, blood banks, and other similar applications.

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: Code 128

Reference Name: code128

Description: Code 128 is a high-density linear barcode symbology defined in ISO/IEC 15417:2007

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: Code 39

Reference Name: code39

Description: Code 39 (also known as Alpha39, Code 3 of 9, Code 3/9, Type 39, USS Code 39, or USD-3) is a variable length, discrete barcode symbology defined in ISO/IEC 16388:2007.

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: Code 93

Reference Name: code93

Description: Code 93 is a barcode symbology designed in 1982 by Intermec to provide a higher density and data security enhancement to Code 39.

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: EAN 13

Reference Name: ean13

Description: European Article Number barcodes (13 digits) are used worldwide for lookup at retail point of sale, but can also be used as numbers for other purposes such as wholesale ordering or accounting.

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: EAN 8

Reference Name: ean8

Description: European Article Number barcodes (8 digits) are used worldwide for lookup at retail point of sale, but can also be used as numbers for other purposes such as wholesale ordering or accounting

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: GS1 Databar

Reference Name: rss14

Description: The family of GS1 Databar barcodes, formerly known as Reduced Space Symbology (RSS-14)

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: IMB

Reference Name: imb

Description: The Intelligent Mail Barcode (IMB) is a 65-bar barcode for use on mail in the United States.

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: ITF

Reference Name: itf

Description: Interleaved 2 of 5 (ITF) is a continuous two-width barcode symbology encoding digits. It is used commercially on 135 film, for ITF-14 barcodes, and on cartons of some products.

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: Pharmacode

Reference Name: pharmacode

Description: Pharmaceutical Binary Code used in the pharmaceutical industry as a packing control system.

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: UPC EAN Extensions

Reference Name: upceanextensions

Description: Detect any UPC-EAN add on text in the result.

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: UPC-A

Reference Name: upca

Description: Universal Product Code - format A, used for tracking commercial items.

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: UPC-E

Reference Name: upce

Description: Universal Product Code - format E, used for tracking commercial items.

Category: 1-D Codes

Read Only: False

Type: Boolean

- Display Name: Aztec

Reference Name: aztec

Description: Aztec-Code is a high-capacity 2D matrix barcode symbology. It can encode ASCII and extended ASCII characters. ISO/IEC 24778:2008 standard defines Aztec-Code.

Category: 2-D Codes

Read Only: False

Type: Boolean

- Display Name: Data Matrix

Reference Name: datamatrix

Description: A Data Matrix is a two-dimensional code consisting of black and white "cells" or dots arranged in either a square or rectangular pattern, also known as a matrix. The information to be encoded can be text or numeric data. Usual data size is from a few bytes up to 1556 bytes.

Category: 2-D Codes

Read Only: False

Type: Boolean

- Display Name: Maxicode

Reference Name: maxicode

Description: MaxiCode is a public domain, machine-readable symbol system originally created by the United Parcel Service (UPS).

Category: 2-D Codes

Read Only: False

Type: Boolean

- Display Name: PDF-417

Reference Name: pdf417

Description: The PDF417 is a 2D barcode. It is based on stacked barcodes.

Category: 2-D Codes

Read Only: False

Type: Boolean

- Display Name: QR Code

Reference Name: qrcode

Description: A QR code (short for " quick-response code ") is a type of two-dimensional matrix barcode.

Category: 2-D Codes

Read Only: False

Type: Boolean

- Display Name: Find All

Reference Name: findall

Description: When enabled, find all barcodes visible in the image. Note, take care when looking for barcodes of differing formats. Some barcode formats are similar and can result in multiple detections with different codes for the same barcode pattern.

Category: Results Settings

Read Only: False

Type: Boolean

- Display Name: Result Color

Reference Name: resultdisplaycolor

Description: The color to use when rendering the result markers

Category: Results Settings

Read Only: False

Type: Enum Gold, Blue, Violet, Red, Black, White

- Display Name: Result Font Size

Reference Name: resultfontsize

Description: The font size of the barcode displayed in the image

Category: Results Settings

Read Only: False

Type: Integer

- Display Name: Show Barcode Result

Reference Name: showbarcodetext

Description: Show the barcode text as a result in the image display.

Category: Results Settings

Read Only: False

Type: Boolean

- Display Name: Show Results

Reference Name: showresults

Description: Alters how the results of a tool is graphically displayed.  Each vision tool has a default method for display, e.g. a line or a frame.   This property allows the graphical display to be changed.  For example, a Fixed Frame Tool is normally displayed as a reference frame.  However, if it is being used as a datum line or a point, its display can be changed to a line or a point for visual clarity.  This property does not affect the actual results of the tool.

Category: Results Settings

Read Only: False

Type: Enum NONE, POINT, LINE, FRAME, ARC

# Appendix – Light Control Tool Properties

- Display Name: Camera Number

Reference Name: cameranumber

Description: The camera the tool executes against

Category: Identity

Read Only: True

Type: Integer

- Display Name: Name

Reference Name: name

Description: The name of the tool instance

Category: Identity

Read Only: False

Type: String

- Display Name: Brightness %

Reference Name: brightness

Description: The brightness level from 0-100

Category: Operation

Read Only: False

Type: Integer

- Display Name: LED Bank

Reference Name: bank

Description: The bank of LEDs to modify, starting at bank 1. If the back is set to zero, the default bank will be used.

Category: Operation

Read Only: False

Type: Integer

- Display Name: Time Delay (ms)

Reference Name: delay

Description: The delay in ms

Category: Operation

Read Only: False

Type: Integer